Complete QFN Reliability-Test Allocation Plan

Temperature–RH matrix, device allocation, hydrogen runs, schedule, and count verification

| Metric | Value |
| --- | --- |
| Equivalent experimental units | 74 |
| Physical QFN packages | 75 |
| Packages in the hydrogen group | 50 |
| Packages in the control group | 25 |
| Capacity per run | 14 for 25°C H₂; 12 standard; 13 in Control Run 2 |

Operational note: 1234 and 1252 are two physical 2T packages used together as one equivalent Type A unit in the no-hydrogen control group.

1. Master Allocation Table

Temperatures: 25°C and 35°C

| Temperature | RH | ParC only | Pd only — with H₂ | Pd¦ParC without H₂ | Pd¦ParC devices with H₂ |
| --- | --- | --- | --- | --- | --- |
| 25°C | 10% | 1000 | 1195 | 1408 (Type B) | 1236 (Type A) 1250 (Type A) 1386 (Type B) |
| 25°C | 30% | 1003 | 1196 | 1376 (Type B) | 1239 (Type A) 1241 (Type A) 1257 (Type A) 1258 (Type A) |
| 25°C | 50% | 1011 | 1197 | 1281 (Type A) | 1242 (Type A) 1243 (Type A) 1388 (Type B) 1394 (Type B) |
| 35°C | 10% | 1012 | 1198 | 1387 (Type B) | 1244 (Type A) 1245 (Type A) 1380 (Type B) |
| 35°C | 30% | 1013 | 1199 | 1283 (Type A) | 1247 (Type A) 1248 (Type A) 1389 (Type B) |
| 35°C | 50% | 1014 | 1200 | 1384 (Type B) | 1255 (Type A) 1256 (Type A) 1379 (Type B) |

1. Master Allocation Table — Continued

| Temperature | RH | ParC only | Pd only — with H₂ | Pd¦ParC without H₂ | Pd¦ParC devices with H₂ |
| --- | --- | --- | --- | --- | --- |
| 45°C | 10% | 1015 | 1201 | 1285 (Type A) | 1260 (Type A) 1261 (Type A) 1393 (Type B) |
| 45°C | 30% | 1016 | 1202 | 1390 (Type B) | 1263 (Type A) 1264 (Type A) 1392 (Type B) |
| 45°C | 50% | 1017 | 1203 | 1293 (Type A) | 1266 (Type A) 1268 (Type A) 1391 (Type B) |
| 60°C | 10% | 1018 | 1204 | 1272 (Type A) | 1271 (Type A) 1274 (Type A) 1328 (Type B) |
| 60°C | 30% | 1019 | 1205 | 1234 (Type A, 2T) 1252 (Type A, 2T) | 1277 (Type A) 1278 (Type A) 1377 (Type B) |
| 60°C | 50% | 1020 | 1206 | 1397 (Type B) | 1279 (Type A) 1280 (Type A) 1385 (Type B) |

The standard condition contains six equivalent experimental units. Two additional Pd|ParC H₂ devices were added at 25°C: device 1257 at 30% RH and device 1394 at 50% RH. The 60°C/30% condition contains seven physical packages because of the 2T pair.

2. Four Hydrogen Runs

Each run covers all three RH levels at one temperature. The 25°C run uses 14 physical packages (three Pd and eleven Pd|ParC); each of the other temperature runs uses 12 physical packages (three Pd and nine Pd|ParC).

Run 1 — 25°C

| RH | Pd | Pd¦ParC with H₂ | Physical packages |
| --- | --- | --- | --- |
| 10% | 1195 | 1236 (Type A) 1250 (Type A) 1386 (Type B) | 4 packages |
| 30% | 1196 | 1239 (Type A) 1241 (Type A) 1257 (Type A) 1258 (Type A) | 5 packages |
| 50% | 1197 | 1242 (Type A) 1243 (Type A) 1388 (Type B) 1394 (Type B) | 5 packages |
| Total | 3 Pd devices | 11 Pd¦ParC devices | 14 packages |

Run 2 — 35°C

| RH | Pd | Pd¦ParC with H₂ | Physical packages |
| --- | --- | --- | --- |
| 10% | 1198 | 1244 (Type A) 1245 (Type A) 1380 (Type B) | 4 packages |
| 30% | 1199 | 1247 (Type A) 1248 (Type A) 1389 (Type B) | 4 packages |
| 50% | 1200 | 1255 (Type A) 1256 (Type A) 1379 (Type B) | 4 packages |
| Total | 3 Pd devices | 9 Pd¦ParC devices | 12 packages |

Run 3 — 45°C

| RH | Pd | Pd¦ParC with H₂ | Physical packages |
| --- | --- | --- | --- |
| 10% | 1201 | 1260 (Type A) 1261 (Type A) 1393 (Type B) | 4 packages |
| 30% | 1202 | 1263 (Type A) 1264 (Type A) 1392 (Type B) | 4 packages |
| 50% | 1203 | 1266 (Type A) 1268 (Type A) 1391 (Type B) | 4 packages |
| Total | 3 Pd devices | 9 Pd¦ParC devices | 12 packages |

Run 4 — 60°C

| RH | Pd | Pd¦ParC with H₂ | Physical packages |
| --- | --- | --- | --- |
| 10% | 1204 | 1271 (Type A) 1274 (Type A) 1328 (Type B) | 4 packages |
| 30% | 1205 | 1277 (Type A) 1278 (Type A) 1377 (Type B) | 4 packages |
| 50% | 1206 | 1279 (Type A) 1280 (Type A) 1385 (Type B) | 4 packages |
| Total | 3 Pd devices | 9 Pd¦ParC devices | 12 packages |

3. Measurement-Card Assignment

The standard twelve-card structure is retained across all four temperature runs. For the 25°C H₂ run only, cards 117 and 175 are added, increasing capacity to 14 physical packages. All cards are reusable measurement platforms, so they may be reassigned in later runs.

| RH | Card | 25°C | 35°C | 45°C | 60°C |
| --- | --- | --- | --- | --- | --- |
| 10% | 119 | 1195 (Pd) | 1198 (Pd) | 1201 (Pd) | 1204 (Pd) |
| 10% | 120 | 1236 (Type A) | 1244 (Type A) | 1260 (Type A) | 1271 (Type A) |
| 10% | 124 | 1250 (Type A) | 1245 (Type A) | 1261 (Type A) | 1274 (Type A) |
| 10% | 125 | 1386 (Type B) | 1380 (Type B) | 1393 (Type B) | 1328 (Type B) |
| 30% | 166 | 1196 (Pd) | 1199 (Pd) | 1202 (Pd) | 1205 (Pd) |
| 30% | 167 | 1239 (Type A) | 1247 (Type A) | 1263 (Type A) | 1277 (Type A) |
| 30% | 168 | 1241 (Type A) | 1248 (Type A) | 1264 (Type A) | 1278 (Type A) |
| 30% | 169 | 1258 (Type A) | 1389 (Type B) | 1392 (Type B) | 1377 (Type B) |
| 50% | 170 | 1197 (Pd) | 1200 (Pd) | 1203 (Pd) | 1206 (Pd) |
| 50% | 171 | 1242 (Type A) | 1255 (Type A) | 1266 (Type A) | 1279 (Type A) |
| 50% | 172 | 1243 (Type A) | 1256 (Type A) | 1268 (Type A) | 1280 (Type A) |
| 50% | 173 | 1388 (Type B) | 1379 (Type B) | 1391 (Type B) | 1385 (Type B) |

Additional Cards — 25°C H₂ Run Only

| RH | Card | Assigned device |
| --- | --- | --- |
| 30% | 117 | 1257 (Type A) |
| 50% | 175 | 1394 (Type B) |

Operational mapping: card 117 → device 1257 at 25°C/30% RH; card 175 → device 1394 at 25°C/50% RH. Within each RH condition, devices are listed in ascending numerical order in the hydrogen-run table. Card 117 can subsequently be reused in Control Run 2.

3.1 Control-Test Card Assignment

The control DOE contains 24 equivalent tests: 12 ParC-only controls and 12 Pd|ParC controls without H₂. Because devices 1234 and 1252 are two physical 2T packages used together as one equivalent unit, the control plan contains 25 physical packages.

| Card | Control Run 1 | Control Run 2 |
| --- | --- | --- |
| 119 | 1000 (ParC; 25°C/10% RH) | 1015 (ParC; 45°C/10% RH) |
| 120 | 1408 (Pd¦ParC Type B; 25°C/10% RH) | 1285 (Pd¦ParC Type A; 45°C/10% RH) |
| 124 | 1003 (ParC; 25°C/30% RH) | 1016 (ParC; 45°C/30% RH) |
| 125 | 1376 (Pd¦ParC Type B; 25°C/30% RH) | 1390 (Pd¦ParC Type B; 45°C/30% RH) |
| 166 | 1011 (ParC; 25°C/50% RH) | 1017 (ParC; 45°C/50% RH) |
| 167 | 1281 (Pd¦ParC Type A; 25°C/50% RH) | 1293 (Pd¦ParC Type A; 45°C/50% RH) |
| 168 | 1012 (ParC; 35°C/10% RH) | 1018 (ParC; 60°C/10% RH) |
| 169 | 1387 (Pd¦ParC Type B; 35°C/10% RH) | 1272 (Pd¦ParC Type A; 60°C/10% RH) |
| 170 | 1013 (ParC; 35°C/30% RH) | 1019 (ParC; 60°C/30% RH) |
| 171 | 1283 (Pd¦ParC Type A; 35°C/30% RH) | 1234 (Pd¦ParC Type A, 2T; 60°C/30% RH) |
| 172 | 1014 (ParC; 35°C/50% RH) | 1252 (Pd¦ParC Type A, 2T; 60°C/30% RH) |
| 173 | 1384 (Pd¦ParC Type B; 35°C/50% RH) | 1020 (ParC; 60°C/50% RH) |
| 117 | Not used | 1397 (Pd¦ParC Type B; 60°C/50% RH) |

Capacity summary: Control Run 1 = 12 physical packages / 12 equivalent tests; Control Run 2 = 13 physical packages / 12 equivalent tests. Card 117 is reused as the thirteenth platform in Control Run 2, so no completion run is required.

4. Daily Schedule

| Day | Run | Content | Physical packages |
| --- | --- | --- | --- |
| Day 1 | 1 | Hydrogen — all RH conditions at 25°C; cards 117 and 175 added | 14 |
| Day 1 | 2 | Hydrogen — all RH conditions at 35°C | 12 |
| Day 1 | 3 | Hydrogen — all RH conditions at 45°C | 12 |
| Day 1 | 4 | Hydrogen — all RH conditions at 60°C | 12 |
| Day 2 | 1 | Controls — all 25°C and 35°C conditions | 12 |
| Day 2 | 2 | Controls — all 45°C and 60°C conditions; card 117 reused for device 1397 | 13 |

Control-Run Details

| Run | Devices | Physical packages |
| --- | --- | --- |
| Control 1 | 1000, 1408, 1003, 1376, 1011, 1281, 1012, 1387, 1013, 1283, 1014, 1384 | 12 |
| Control 2 | 1015, 1285, 1016, 1390, 1017, 1293, 1018, 1272, 1019, 1234, 1252, 1020, 1397 | 13 |

Control Run 2 reuses card 117 as the thirteenth platform for device 1397. This does not conflict with its earlier 25°C H₂ assignment because the devices are moved between reusable measurement platforms.

5. Type B Balance

Balance by Temperature

| Temperature | Type B with H₂ | Type B without H₂ | Total Type B |
| --- | --- | --- | --- |
| 25°C | 3 | 2 | 5 |
| 35°C | 3 | 2 | 5 |
| 45°C | 3 | 1 | 4 |
| 60°C | 3 | 1 | 4 |
| Total | 12 | 6 | 18 |

With device 1394 added at 25°C/50% RH, the 18 Type B devices are distributed across the four temperatures as 5, 5, 4, and 4.

Balance by RH

| RH | Type B with H₂ | Type B without H₂ | Total Type B |
| --- | --- | --- | --- |
| 10% | 4 | 2 | 6 |
| 30% | 3 | 2 | 5 |
| 50% | 5 | 2 | 7 |
| Total | 12 | 6 | 18 |

The current Type B distribution across RH levels is 6, 5, and 7 for 10%, 30%, and 50% RH, respectively.

Balance by Experimental Group

| Group | Type A — equivalent units | Type B | Total Pd¦ParC |
| --- | --- | --- | --- |
| With H₂ | 26 | 12 | 38 |
| Without H₂ | 6 | 6 | 12 |
| Total | 32 | 18 | 50 |

6. Count and Allocation Verification

| Device type / check | Original target | Current allocation | Status |
| --- | --- | --- | --- |
| ParC only | 12 | 12 | Pass |
| Pd only | 12 | 12 | Pass |
| Full Type A devices | 29 | 31 | Deviation |
| Partial Type A devices | 1234 + 1252 | Allocated together | Pass |
| Equivalent Type A units | 30 | 32 | Deviation |
| Type B devices | 18 | 18 | Pass |
| Pd¦ParC with H₂ | 36 | 38 | Expanded |
| Pd¦ParC without H₂ | 12 units | 12 units / 13 packages | Pass |
| Packages in hydrogen runs | 12 per run | 14, 12, 12, 12 | Expanded |

Verification Summary

| Control point | Result |
| --- | --- |
| Duplicates | No duplicate device numbers were found |
| Updated devices | Added 1257 (Type A) at 25°C/30% RH and 1394 (Type B) at 25°C/50% RH |
| Type composition | Current allocation contains 32 equivalent Type A units and 18 Type B devices |
| Physical packages | All 75 packages are allocated exactly once |
| Type B in hydrogen group | 12 Type B devices are allocated to H₂; 25°C/50% RH contains two Type B devices |
| H₂ card additions | Card 117 → device 1257; card 175 → device 1394 in the 25°C run |
| Hydrogen-run capacity | Physical-package counts are 14, 12, 12, and 12 for 25°C, 35°C, 45°C, and 60°C |
| Control-test card allocation | 24 equivalent control tests are assigned across 25 physical packages: 12 + 13 card slots |
| Platform reuse | Card 117 is reused later in Control Run 2; device movement prevents any scheduling conflict |